Practical Lab Book MCA 2025-2026 SEM I

Subject – Internet Of Things

Submitted by-	Guided By-

SRN:31242488

Tushar Betala	        Dr. Bharati Pandurang Kawade

INDEX -1

| Sr. No. | Problem Statement | Signature |
| --- | --- | --- |
| 01 | To interface Light Emitting Djode(LED)- Blinking LED. |  |
| 02 | To interface Button and LED – LED blinking when button is pressed. |  |
| 03 | To interface a PIR Sensor with Arduino to Drive an LED Indicator. |  |
| 04 | To use the DHT11 temperature & humidity sensor with Arduino. |  |
| 05 | To interface the LM35 temperature sensor with Arduino. . |  |
| 06 | To interface the Traffic Signal Simulation with Arduino. |  |
| 07 | To interface the Ultrasonic Sensor with Arduino. |  |
| 08 | To interface 7 segments step by step in Arduino. |  |
| 09 | To interface a 4×4 matrix keypad with Arduino and develop a program to detect and display the pressed key using the serial monitor. |  |
| 10 | To interface an LCD display with Arduino. |  |

Practical 1

- To interface Light Emitting Diode(LED)- Blinking LED.

Code:

const int ledPin = 13; // Pin connected to the LED

void setup() {

pinMode(ledPin, OUTPUT); // Set the LED pin as an output

}

void loop() {

digitalWrite(ledPin, HIGH); // Turn on the LED

delay(1000); // Wait for 1 second

digitalWrite(ledPin, LOW); // Turn off the LED

delay(1000); // Wait for 1 second

}

Output :

| Turn Off the LED | Turn On the LED |
| --- | --- |

- To interface Button and LED – LED blinking when button is pressed.

Code:

const int buttonPin = 2; // Pin connected to the button

const int ledPin = 13; // Pin connected to the LED

int buttonState = 0; // Variable to store the button state

void setup() {

pinMode(ledPin, OUTPUT); // Set the LED pin as an output

pinMode(buttonPin, INPUT); // Set the button pin as an input

}

void loop() {

buttonState = digitalRead(buttonPin); // Read the state of the button (HIGH or LOW)

// Check if the button is pressed (buttonState is LOW)

if (buttonState == LOW) {

digitalWrite(ledPin, HIGH); // Turn on the LED

delay(500); // Wait for 500 milliseconds (0.5 seconds)

digitalWrite(ledPin, LOW); // Turn off the LED

delay(500); // Wait for another 500 milliseconds

}

}

Output :

Turn Off the LED	                                           Turn On the LED

- To interface a PIR Sensor with Arduino to Drive an LED Indicator.

Code:

int pirPin = 2;       // PIR sensor OUT pin

int ledPin = 4;      // LED pin

int pirState=0;

void setup() {

pinMode(pirPin, INPUT);   // Set PIR pin as input

pinMode(ledPin, OUTPUT);  // Set LED pin as output

Serial.begin(9600);       // Start serial communication

}

void loop() {

int pirState = digitalRead(pirPin);  // Read PIR state

if (pirState == HIGH) {

digitalWrite(ledPin, HIGH);  // Turn LED on

Serial.println("Motion Detected!");

} else {

digitalWrite(ledPin, LOW);   // Turn LED off

}

delay(1000);  // Small delay to stabilize reading

}

Output :

- To use the DHT11 temperature & humidity sensor with Arduino.

Code:

#include "DHT.h"   // Include DHT library

#define DHTPIN 2     // Pin connected to DATA

#define DHTTYPE DHT11   // Define sensor type (DHT11 or DHT22)

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(9600);

dht.begin();

Serial.println("DHT11 Sensor Test");

}

void loop() {

// Wait a few seconds between measurements

delay(2000);

float h = dht.readHumidity();

float t = dht.readTemperature(); // Celsius

// Check if reading failed

if (isnan(h) || isnan(t)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

Serial.print("Humidity: ");

Serial.print(h);

Serial.print(" %\t");

Serial.print("Temperature: ");

Serial.print(t);

Serial.println(" °C");

}

Output :

- To interface the LM35 temperature sensor with Arduino.

Code:

int sensorPin = A0;   // LM35 output connected to A0

float sensorValue;	// to store ADC value

float temperature;	// calculated temperature

void setup() {

Serial.begin(9600);

}

void loop() {

sensorValue = analogRead(sensorPin);   // read analog value (0-1023)

// Convert to voltage (Arduino is 5V with 10-bit ADC)

float voltage = sensorValue * (5.0 / 1023.0);

// LM35 gives 10mV per °C → 0.01V = 1°C

temperature = voltage * 100;

Serial.print("Temperature: ");

Serial.print(temperature);

Serial.println(" °C");

delay(1000); // read once per second

}

Output :

- To interface the Traffic Signal Simulation with Arduino.

Code:

int redLed = 8;

int yellowLed = 9;

int greenLed = 10;

void setup() {

pinMode(redLed, OUTPUT);

pinMode(yellowLed, OUTPUT);

pinMode(greenLed, OUTPUT);

}

void loop() {

// Red ON (Stop)

digitalWrite(redLed, HIGH);

digitalWrite(yellowLed, LOW);

digitalWrite(greenLed, LOW);

delay(5000);  // 5 seconds

// Green ON (Go)

digitalWrite(redLed, LOW);

digitalWrite(yellowLed, LOW);

digitalWrite(greenLed, HIGH);

delay(5000);  // 5 seconds

// Yellow ON (Caution)

digitalWrite(redLed, LOW);

digitalWrite(yellowLed, HIGH);

digitalWrite(greenLed, LOW);

delay(2000);  // 2 seconds

}

Output :

| RED GREEN | YELLOW |
| --- | --- |

- To interface the Ultrasonic Sensor with Arduino.

Code:

// HC-SR04 simple example (Trig=9, Echo=10)

const int trigPin = 9;

const int echoPin = 10;

unsigned long duration;

float distanceCm;

void setup() {

Serial.begin(9600);

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

}

void loop() {

// Make sure Trig is LOW

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

// Send 10 microsecond pulse to trigger

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

// Read the time for echo (timeout 30000 us ~ 5 m)

duration = pulseIn(echoPin, HIGH, 30000);

if (duration == 0) {

Serial.println("Out of range");

} else {

// distance in cm: duration (us) * 0.01715

distanceCm = duration * 0.01715;

Serial.print("Distance: ");

Serial.print(distanceCm);

Serial.println(" cm");

}

delay(200); // 5 readings/second

}

Output :

- To interface 7 segments step by step in Arduino.

Code:

// Pin mapping for segments a-g

int segPins[7] = {2, 3, 4, 5, 6, 7, 8};

// Digit patterns for 0-9 (1 = ON, 0 = OFF)

byte digits[10][7] = {

{1,1,1,1,1,1,0}, // 0

{0,1,1,0,0,0,0}, // 1

{1,1,0,1,1,0,1}, // 2

{1,1,1,1,0,0,1}, // 3

{0,1,1,0,0,1,1}, // 4

{1,0,1,1,0,1,1}, // 5

{1,0,1,1,1,1,1}, // 6

{1,1,1,0,0,0,0}, // 7

{1,1,1,1,1,1,1}, // 8

{1,1,1,1,0,1,1}  // 9

};

void setup() {

for (int i = 0; i < 7; i++) {

pinMode(segPins[i], OUTPUT);

}

}

void loop() {

for (int num = 0; num < 10; num++) {

showDigit(num);

delay(1000); // 1 second delay

}

}

void showDigit(int num) {

for (int i = 0; i < 7; i++) {

digitalWrite(segPins[i], digits[num][i]);

}

}

Output :

- To interface a 4×4 matrix keypad with Arduino and develop a program to detect and display the pressed key using the serial monitor.

Code:

#include <Keypad.h>

const byte ROWS = 4;

const byte COLS = 4;

char keys[ROWS][COLS] = {

{'1','2','3','A'},

{'4','5','6','B'},

{'7','8','9','C'},

{'*','0','#','D'}

};

// change these pin numbers to match your wiring

byte rowPins[ROWS] = {9, 8, 7, 6};    // R0, R1, R2, R3

byte colPins[COLS] = {5, 4, 3, 2};    // C0, C1, C2, C3

Keypad keypad = Keypad( makeKeymap(keys), rowPins, colPins, ROWS, COLS );

void setup() {

Serial.begin(9600);

}

void loop() {

char k = keypad.getKey();

if (k) {

Serial.print("Key: ");

Serial.println(k);

}

}

Output :

- To interface an LCD display with Arduino.

Code:

#include <LiquidCrystal.h>;

const int rs = 12, en = 11, d4 = 5, d5 = 4, d6 = 3, d7 = 2, ct=9;

LiquidCrystal mylcd(rs, en, d4, d5, d6, d7);

void setup() {

analogWrite(ct,50);

//ct- this sets the brightness level. Sets PWM brightness for the LCD backlight (50/255 ≈ 20% brightness).

mylcd.begin(16, 2);  // Initializes the LCD with 16 columns and 2 rows.

mylcd.print("Hello, Arduino!");   // Displays the text “Welcome” on the first line of the LCD.

}

void loop()

{

}

Output :